Values calculated after getting through the Fractional Distillation Process shown in the Diagram:-

- DCOL-1 (Distillation Column)

    - Condenser Pressure Drop = 0 Pa

    - Condenser Duty = 1103.98 kW

    - Reboiler Duty = -1219.73 kw

    - Number of Stages = 20

    - Column Pressure Drop = 0 Pa

    - Condenser_Specification_Value = 2.5

    - Reboiler_Specification_Value = 2

    - Global_Stage_Efficiency = N/D

    - Condenser_Calculated_Value = 2.5

    - Reboiler_Calculated_Value = 2

    - Estimated_Height = 11000 mm

    - Estimated Diameter = 1400.01 mm

- Feed (Material Stream)

  - Temperature = 298.15 K

  - Pressure = 101325 Pa

  - Mass Flow = 1Kg/s

  - Molar Flow = 11.7474 mol/s

  - Volumetric Flow = 0.00115109 m3/s

  - Molar Enthalpy (Mixture) = -34854.6 kJ/kmol

  - Molar Entropy (Mixture) = -89.3434 kJ/[Kmol.K]

  - Molar Fraction (Vapor) = 0

  - Phases = L

  - Vapor : Benzene = 0.7706593, Tolune = 0.2293407

  - Energy Flow = 372.045 kW

FRACTIONAL DISTILLATION

DWSIM Flowsheet Diagram

- Distillate (Material Stream)

  - Temperature = 362.232 K

  - Pressure = 101325 Pa

  - Mass Flow = 0.815727 kg/s

  - Molar Flow = 9.74741 mol/s

  - Volumetric Flow = 0.00101451 m3/s

  - Molar Enthalpy (Mixture) = -25347.6 kJ/kmol

  - Molar Entropy (Vapor) = 0

  - Phases = L

  - Liquid 1 : Benzene = 0.60255992, Tolune = 0.39744008

  - Energy Flow = 434.073 kW

- Bottoms (Material Stream)

  - Temperature = 383.814 K

  - Pressure = 101325 Pa

  - Mass Flow = 0.184273 kg/s

  - Molar Flow = 2 mol/s

  - Volumetric Flow = 0.000236356 m3/s

  - Molar Enthalpy (Mixture) = -23311.5 kJ/kmol

  - Molar Entropy (Mixture) = -57.086 kJ/[kmol.K]

  - Molar Fraction (Vapor) = 0

  - Phases = L

  - Liquid 1 : Benzene = 0.00015346642, Tolune = 0.9984653

  - Energy Flow = 53.727 kW

- E1(Energy Stream) = 1103.98 kW

- E2(Energy Stream) = 1219.73 kW